# Equations and Variable Descriptions
Hearing the Room Through the Shape of the Drum

# Surface Vibration Model

## General wave equation

ρ∂2u∂t2+c(x)∂u∂t-T∇2u+D∇4u=f(x,t)

Description of terms:

- u(x,t): out-of-plane displacement of the surface at spatial location x and time t.

- ρ: areal mass density of the surface.

- c(x): spatially dependent damping coefficient.

- T: in-plane tension of the surface, dominant for membranes.

- D: flexural rigidity, dominant for thin plates.

- ∇2u: Laplacian of the displacement, related to membrane-like wave behavior.

- ∇4u: biharmonic operator, related to the bending behavior of plates.

- f(x,t): external forcing per unit area, induced by acoustic pressure.

For membranes:

D=0

For thin plates under negligible tension:

T=0

# Modal Decomposition

## Expansion into vibration modes

u(x,t)=k=1Kϕk(x)qk(t)

Description of terms:

- ϕk(x): spatial eigenfunction, also called the k-th mode shape.

- qk(t): temporal modal coordinate of the k-th mode.

- K: number of modes used in the approximation.

## Second-order modal oscillator

qk(t)+2ζkωkqk(t)+ωk2qk(t)=αkp(t)+ηk(t)

Description of terms:

- qk(t): temporal activation of the k-th mode.

- qk(t): first temporal derivative, modal velocity.

- qk(t): second temporal derivative, modal acceleration.

- ωk: natural angular frequency of the k-th mode.

- ζk: damping ratio of the k-th mode.

- αk: coupling coefficient between acoustic pressure and the k-th mode.

- p(t): acoustic pressure signal driving the surface.

- ηk(t): noise or modeling error for the k-th mode.

# Frequency-Domain Modal Transfer Function

## Modal response in the frequency domain

Qk(ω)=Gk(ω)P(ω)

Description of terms:

- Qk(ω): Fourier transform of the modal coordinate qk(t).

- P(ω): Fourier transform of the acoustic pressure p(t).

- Gk(ω): transfer function of the k-th vibration mode.

- ω: angular frequency.

## Second-order oscillator transfer function

Gk(ω)=αk-ω2+j2ζkωkω+ωk2

Description of terms:

- Gk(ω): frequency response of the k-th mode.

- αk: modal coupling coefficient.

- j: imaginary unit.

- ζk: damping ratio.

- ωk: natural angular frequency of the mode.

- ω: evaluated angular frequency.

# Surface Displacement as a Modal Convolution

u(x,t)=k=1Kϕk(x)p(t)*gk(t)

Description of terms:

- gk(t): impulse response of the k-th mode.

- *: convolution operator.

- p(t)*gk(t): acoustic pressure filtered by the mechanical response of the k-th mode.

# Sound Pressure and Source Signal

p(t)=γs(t)

Description of terms:

- s(t)∈[-1,1]: dimensionless source audio signal.

- p(t): physical acoustic pressure in Pascals.

- γ: scale factor accounting for source amplitude and acoustic attenuation.

# Speckle-Based Vibration Measurements

## Speckle shifts as surface gradients

v(xn,t)=β∇xu(xn,t)

Description of terms:

- v(xn,t)∈R2: measured two-axis speckle shift at surface point xn.

- xn: the n-th measured surface point.

- β: optical transfer scale factor.

- ∇xu(xn,t): spatial gradient of the surface displacement.

## Forward model from sound to measured speckle shifts

v(xn,t)≈γβk=1K∇ϕk(xn)s(t)*gk(t)+η(xn,t)

Description of terms:

- v(xn,t): measured two-axis vibration signal at point xn.

- ∇ϕk(xn): spatial gradient of the k-th mode shape at point xn.

- s(t): latent source sound signal to be recovered.

- gk(t): impulse response of the k-th mode.

- η(xn,t): measurement noise or modeling error.

- γβ: combined acoustic and optical scale factor.

# Fourier Transform of the Measured Vibrations

V(xn,ω)≡Fv(xn,t),  V(xn,ω)∈C2

Description of terms:

- F{⋅}: Fourier transform operator.

- V(xn,ω): complex frequency-domain representation of the two-axis vibration signal.

- C2: two-dimensional complex vector space.

# Mode Frequency Estimation

ωkk=1K←robustfindpeaksV(xn,ω)

Description of terms:

- ωk: estimated natural frequency of the k-th mode.

- robustfindpeaks(⋅): robust peak detection procedure used to identify modal frequencies.

- |V(xn,ω)|: magnitude spectrum of the measured vibration signal.

# Mode Shape Gradient Estimation

∇ϕk(xn)=ReV(xn,ωk)⋅V1(x0,ωk)*En,aV(xn,ωk)⋅V1(x0,ωk)

Description of terms:

- ∇ϕk(xn): estimated gradient of the k-th mode shape at point xn.

- Re{⋅}: real part operator.

- V(xn,ωk): measured vibration spectrum at the estimated modal frequency.

- V1(x0,ωk): first-axis vibration measurement at reference point x0.

- (⋅)*: complex conjugate.

- En,a[⋅]: average over all points n and vibration axes a.

# Sound Recovery Optimization

sinv(t)=argmins(t),αkv(xn,t)-k=1K∇ϕk(xn)s(t)*gk(t)22+λs(t)22

Description of terms:

- sinv(t): recovered source sound signal.

- s(t): latent sound signal optimized during recovery.

- αk: modal coupling coefficient optimized jointly with the signal.

- ∇ϕk(xn): estimated gradient of the k-th mode shape.

- gk(t): estimated impulse response of the k-th mode.

- λ: regularization weight.

- s(t): temporal derivative of the recovered sound signal.

- ∥⋅∥22: squared Euclidean norm.

## Impulse response from the transfer function

gk(t)←iFFTGk(ω)

Description of terms:

- iFFT(⋅): inverse Fast Fourier Transform.

- Gk(ω): analytical modal transfer function.

- gk(t): estimated time-domain modal impulse response.

# Robust Mode Estimation

## Spectral standard deviation across surface points

σ(ω)=stdnV(xn,ω)

Description of terms:

- σ(ω): standard deviation of spectral magnitudes across surface points.

- stdn(⋅): standard deviation over measured points n.

- |V(xn,ω)|: magnitude spectrum of the vibration signal at point xn.

## Total variation of a mode-shape gradient

TV∇ϕk=x,y​∂∂x∇ϕk(x,y)2+∂∂y∇ϕk(x,y)2

Description of terms:

- TV(∇ϕk): total variation of the gradient field of the k-th mode shape.

- ∂∂x∇ϕk(x,y): spatial derivative of the mode-gradient field along the x direction.

- ∂∂y∇ϕk(x,y): spatial derivative of the mode-gradient field along the y direction.

- ∥⋅∥2: Euclidean norm.

# Calibration-Based Baseline

## Reference convolution model

varef(xn,t)≈sref(t)*hn,a(t)

Description of terms:

- varef(xn,t): reference vibration signal at point xn and axis a.

- sref(t): known reference audio signal used for calibration.

- hn,a(t): impulse response from the reference sound to point xn and vibration axis a.

- a∈{1,2}: vibration axis index.

## Inverse filter estimation

hn,a-1(t)=argminh(t)varef(xn,t)*h(t)-sref(t)22

Description of terms:

- hn,a-1(t): estimated inverse filter for point xn and axis a.

- h(t): candidate inverse filter optimized by least squares.

- varef(xn,t)*h(t): calibrated reconstruction of the reference signal.

## Recovered signal using calibrated inverse filters

snew(t)=12Na=12n=1Nvanew(xn,t)*hn,a-1(t)

Description of terms:

- snew(t): recovered signal for a new measurement.

- vanew(xn,t): new measured vibration signal at point xn and axis a.

- hn,a-1(t): calibrated inverse filter.

- N: number of measured surface points.

- 2N: total number of measurement channels, since each point has two vibration axes.

# Implementation Constants

ζk=0.01

Description of term:

- ζk: damping ratio fixed to 0.01 in the experiments.

λ=1

Description of term:

- λ: regularization weight used in the sound recovery optimization.

Tconv=4096

Description of term:

- Tconv: number of taps used for the inverse filters in the calibration baseline.